Issues caused by poorly designed circuits

- Difficulty debugging: tracing will be difficult

Breadboard:

- power rail,

- ground rail,

- DIP IC Break

- conductive strips

Color coding wires (Suggestions)

- Be sure to be consistent

- Black: ground

- Red: power 5V

- White: power higher voltage

- Organe: communication Tx

- Green: communication Rx

- Blue: analog measurement

- Purple: PWM output

Ground

- Voltage is only measured as the difference in potential between two points

- Typically, a 0V point in a circuit is defined as the “ground

- Having a common ground is important

- You can have separate power sources for different parts of the circuit, but make sure you have a common ground relative to each other. Connect them and short them

  - If you have multiple grounds, you won’t know the voltage of the two grounds relative to each other

  - You might accidentally give more than 5V of power for the arduino analog pins, which will break the pins

Noise

- Undesirable random oscillation of the voltage/current, very quick changes

- Can come from various sources: Switched power supplies, PWM signals, microcontroller

Electromagnetic interference

- Basics

  - A changing magnetic field induces a current in nearby wires, and vice versa

- Any wire with changing current can generate or pick up EM waves, other wires will pick up this noise

Interference in servomotors

- Changing angle based on width of pulse (PWM signal)

- If the current is changed due to EM waves, servomotors will have a hard time interpreting the signal

What can we do about electromagnetic interference problems

- Twisted pair wiring

  - Opposite induced currents cancel each other

- Shielding

  - Keeps noise either in or out

  - Can shield both wires/cables and integrated circuits

  - Shielding should be grounded (dissipate any energy if they pick up)

  - Use foil, braid, or a combination of them to shield

- Ferrites, time averaging (only in some cases)

  - Ferromagnetic material

  - Wire passes through the core

  - Attenuate high-frequencies (acts as an inductor at high frequency), can be noise or signals

  - Can cause degradation of square wave signals, such as PWM (shaping square wave signals into rounded signals)

  - May not be effective at low frequencies

- Time averaging in code

  - Averaging several discrete measurements to remove noise

  - Only works if noise is “zero-mean”

    - If non-zero mean, e.g. if the voltage is constantly decreasing over time, then you won’t be able to detect it

  - Can cause “smearing”

  - Averaging time is on the same order of magnitude as the signal period

Switching noise

- When using a switched power supply (buck, boost, or buck-boost converter) is used, the rapid on/off of the circuit will cause high frequency oscillations in power lines

  - Due to parasitic line capacitances and inductances

- Can cause problems if the harmonics approach the magnitude of the voltage on the line (wavy voltage)

Mitigating switching noise

- To fix it, we need to use power line filtering

  - Ferrites can be used for this

  - Parallel filtering capacitors (polarized and unpolarized)  can be much smaller and more effective

    - Effectively absorb high frequency noise components

    - Large capacitance values will absorb at lower frequencies

    - Many ICs will specify values of capacitors for proper operation

Voltage and current

- Only dealing with DC in this course

Wires

- Shorter and wider → more resistance

- Stranded: more flexible, generally easier to work with

- Solid core: higher current handling, better heat conduction

- Jumper wire: no soldering needed

Resistors

- Passive component, dissipates power, limited by how much power they can safely dissipate before melting

- Used to limit current draw

- Available as “barrels”, surface mount, etc

- What resistance should I use to draw 100 mA from a 10V power supply, what power dissipation?

Capacitors

- Stores and releases a small amount of charge (F)

- Various types, some polarized, some non-polarized (don’t need to worry about which way to plug in)

- At steady state, act as open circuit, will act to quickly absorb current spikes from transient

Inductor

- As current flows through, generates a magnetic field, that resists changes in current

- Acts to slow down current

- At steady state, acts as a short circuit, acts to block transients and smooth out response to changes

- Combined with capacitor, can form LC circuit

Motors

- Typically will act as a resistor in a circuit in steady state, draw power and dissipate energy

- Has voltage rating, draw amount of current they need (has circuit embedded in it usually to control the circuit)

Switch

- Types: single pole single throw, single pole double throw, double pole double throw (doesn’t have an off state)

- Throw: number of position

Transistor

- Semiconductor, act like switches

- Switch electrically, solid state, no moving parts, incredibly fast response

- Power devices that may take more current than an Arduiino can provide over GPIO

- Many different types

- Still have current/voltage limits, depending on type/size

Relay

- Electromechanical switch: often uses magnetic fields to open/close

- Latching/non-latching available

- Requires more power than a transistor to open/close

- Much greater power handling capability

Diodes

- One-way current controller, prevent current from flowing in one direction

- Characterized for a forward voltage drop, breakdown voltage, and current limit

  - Signal vs. rectifier (power) diode (one used for signal, one used for power)

LED

- Acts as a diode, but emits light when current passes through it

- Follows the same IV curve as a normal diode, forward voltage is normally higher

- Brightness depends on current passing through diode, resistor in the circuit limits current

Programmable DC Power Supplies

- Useful for testing

- Settable voltage and current limits

  - Analog vs. Digital

- More advanced units have separate power on and enable switches: multi-channel and digital control also available

Batteries

- Voltage - nominal operating voltage: more series cells, higher voltage

- Capacity - amp-hours: more parallel cells, higher capacity (Ah)

- Discharge: amperage pulled from battery relative to its total capacity

  - Discharge rate

- Different battery chemistries good for different applications

- Use batteries in series instead of one battery to provide that same voltage, but they are heavy

Linear regular

- Userful power applications

- Provides a constant voltage lower than the supply voltage

- Excess power dissipated as heat

  - Seriously, these can get really hot

  - Pd = (Vin-Vout) Iin

- No switching noise

Switched power supplies

- More efficient than linear regulators

  - Efficiency in the range of 95-98%

  - Don’t heat up as much, can handle higher currents/voltages

  - More complex, larger

- Can both increase or decrease voltage

- Can cause switching noise on power lines

Some basic circuits

- Voltage divider (use Ohm’s law to find voltage drop across different resistors)

- LED dimmer

- Pull-down resistor:

  - used to avoid “floating” digital input pins, keeps the voltage at the appropriate level, 5 when switch is closed, 0V when open

- Pull-up resistor:

  - Same thing as pull-down resistor, but with power and ground reversed

Integrated circuits

- Discrete: discrete components require more space but more flexible

- Integrated circuit has huge reduction in size, and generally higher efficiency and lower cost

- Consider different footprint/package avilable